KERMIT'S MODEM SUPPORT

---   PREPUBLICATION OUTLINE/DRAFT   ---

Copyright (C) 1995, Frank da Cruz and Christine M. Gianone,

All rights reserved.  FOR REVIEW AND COMMENT ONLY; NOT FOR

REDISTRIBUTION IN ANY FORM.

Most recent update: February 8, 1996

THIS IS AN UPDATE TO CHAPTER 3, "GETTING CONNECTED", OF "USING C-KERMIT".

When you give Kermit a DIAL or ANSWER command, it operates your modem using

built-in knowledge of the kind of modem you have, or an external driver for

the modem, or else information that you have supplied for a "user-defined"

modem type.  So it's important that you give Kermit the best possible

information about the modem.  Usually, all you have to do is tell Kermit what

type of modem you have, and Kermit does the rest automatically.  The

command is:

SET MODEM TYPE <name>

Selects a modem type (or NONE for direct serial connections) from Kermit's

built-in database of modems.  The default modem type is NONE.

Kermit's built-in knowledge applies to many types of modems.  Table I shows

the list as of this writing.  "SET MODEM TYPE ?" might display additional

names if this documenent is out of date.

------------------------------------------------------------------------------

Table I - Modem Types

------------------------------------------------------------------------------

Name               Characteristics      Description

------------------------------------------------------------------------------

att-dataport       AT EC DC LF SB       AT&T Dataport

att-dtdm                                AT&T Digital Terminal Data Module

att-isn                                 AT&T ISN Network

att-switched-net   AT                   AT&T Switched-Network Modems

att-7300                                AT&T 7300 (UNIX PC) Internal Modem

boca               AT EC DC LF SB       Boca modems

cermetek                                Cermetek Info-Mate 212A 1200 bps

concord                                 Concord Condor CDS 220 2400 bps

df03-ac                                 Digital Equipment Corp DF03-AC

df100-series                            Digital Equipment Corp DF100 series

df200-series                            Digital Equipment Corp DF200 series

digitel-dt22       ITU                  Digitel DT-22 ITU-T (CCITT) V.25bis

gdc-212a/ed                             General Data Comm 212A/ED

hayes-1200         AT                   Hayes Smartmodem 1200 & compatibles

hayes-2400         AT                   Hayes Smartmodem 2400 & compatibles

hayes-high-speed   AT EC DC LF SB       Hayes Accura, Optima, or Ultra

intel              AT EC DC LF SB       Intel High-Speed FaxModem

itu-t-v25bis       ITU                  ITU-T (CCITT) V.25bis

maxtech            AT EC DC LF SB       MaxTech XM288EA or GVC FAXModem

microcom-at-mode   AT EC DC LF SB       Microcom in Hayes mode (Deskporte)

microcom-sx-mode      EC DC LF SB KS    Microcom in native SX mode

motorola-fastalk   AT EC DC LF SB       Motorola FasTalk II

multitech          AT EC DC LF SB       Multitech MT1432 Series MultiModem II

mwave              AT EC DC LF SB       IBM Mwave PCMCIA

none                                    Direct serial connection

old-telebit        AT EC DC LF SB KS TB Telebit T1000, T1500, T2500, ...

penril                                  Penril

ppi                AT EC DC LF SB       Practical Peripherals modems

racalvadic                     SB       Racal-Vadic VA4492E in native mode

rockwell-v32       AT EC DC LF SB       Rockwell V.32 modems

rockwell-v32bis    AT EC DC LF SB       Rockwell V.32bis modems

rockwell-v34       AT EC DC LF SB       Rockwell V.34 modems

rolm-dcm                                Rolm/Siemens/IBM CBX data phone

supra              AT EC DC LF SB       Supra modems

telebit            AT EC DC LF SB KS TB Telebit T1600, T3000, QBlazer, ...

unknown                                 Unknown modem type

user-defined                            User-defined modem type

usrobotics         AT EC DC LF SB       US Robotics Sportster or Courier

ventel                                  Ven-Tel

zoltrix            AT EC DC LF SB       Zoltrix modems

zoom               AT EC DC LF SB       Zoom modems

zyxel              AT EC DC LF SB       ZyXEL modems

------------------------------------------------------------------------------

For example, "set modem type supra".  If your modem is not in this list,

perhaps it is compatible with one of the built-in types.  Later on, we'll see

how you can add your own modem type to Kermit, in case you have a modem that

is totally different from all the ones that Kermit knows about.

As you can see by comparing this table with Table 3-2 on page 53 of "Using

C-Kermit", lots of new modem types have been added since the manual was

published.

The other big difference is that when you give a SET MODEM TYPE command,

Kermit now fetches all the necessary modem-specific information from its

built-in database and also sets many of its own communication parameters

accordingly.  Thus, in most cases, everything is set up for you automatically

when your choose your modem type, and Kermit works much more smoothly with

modern error-correcting, data-compressing modems.

You can see how this works by giving a SET MODEM TYPE command and then

viewing the information with the SHOW MODEM command.  Example:

[/usr/olga] C-Kermit>set modem type ppi ; Tell Kermit the modem type

[/usr/olga] C-Kermit>show modem         ; View the modem-specific parameters

Modem type: ppi

Practical Peripherals V.22bis or higher with V.42 and V.42bis

Modem carrier:          auto

Modem capabilities:     AT SB EC DC HWFC SWFC

Modem maximum-speed:    115200 bps

Modem error-correction: on

Modem compression:      on

Modem speed-matching:   off (interface speed is locked)

Modem flow-control:     auto

Modem kermit-spoof:     off

Modem escape-character: 43 (= "+")

MODEM COMMANDs (* = set automatically by SET MODEM):

* Init-string:          ATQ0X4N1S37=0S82=128\{13}

* Dial-command:         ATD%s\{13}

* Compression on:       ATS46=2\{13}

* Compression off:      ATS46=0\{13}

* Error-correction on:  AT&Q5S36=7S48=7\{13}

* Error-correction off: AT&Q0S36=0S48=128\{13}

* Autoanswer on:        ATS0=1\{13}

* Autoanswer off:       ATS0=0\{13}

* Hangup-command:       ATQ0H0\{13}

* Hardware-flow:        AT&K3\{13}

* Software-flow:        AT&K4\{13}

* No-flow-control:      AT&K0\{13}

* Pulse:                ATP\{13}

* Tone:                 ATT\{13}

Each of these items is explained below, and each of them can be changed

by you if necessary with the appropriate commands.

The SHOW MODEM command (and the modem table above) include a compact listing

of the modem's capabilities.  These are listed in Table II.

--------------------------------------------------------------------

Table II - Modem capabilities

--------------------------------------------------------------------

AT    Hayes AT command set and responses

ITU   ITU-T (CCITT) V.25bis command set and responses

SB    Speed buffering (interface speed can be locked)

EC    Error correction (MNP or V.42/LAPM)

DC    Data compression (MNP or V.42bis)

HWFC  Hardware flow control (RTS/CTS) (listed in Table I as LF)

SWFC  Local software flow control (Xon/Xoff) (listed in Table I as LF)

KS    Kermit spoofing (modem itself runs Kermit protocol)

TB    Made by Telebit (used internally for simplification)

--------------------------------------------------------------------

Note: LF stands for Local Flow control; that is, modem can engage in

flow control with the computer it is directly connected to.

--------------------------------------------------------------------

IMPORTANT SETTINGS

Look again at this portion of the SHOW MODEM listing (line numbers added):

1. Modem error-correction: on

2. Modem compression:      on

3. Modem speed-matching:   off (interface speed is locked)

4. Modem flow-control:     auto

When you give a SET MODEM TYPE command:

1. If Kermit knows that the modem is capable of error-correction,

Kermit will tell the modem to use it.

2. If Kermit knows that the modem is capable of data-compression,

Kermit will tell the modem to use it.

3. If Kermit knows that the modem is capable of speed-buffering,

Kermit will ASSUME the modem has been configured to use it, and

will, therefore, not change its own interface speed in response to

CONNECT speed reports from the modem.

4. If the modem is capable of local flow-control, Kermit configures it

to use the same kind of flow as Kermit itself is set to use.

Kermit does NOT, however, set any of the following automatically:

1. Communication device

2. Speed of the communication device

3. Parity or terminal bytesize

4. Duplex

So you should take care to ensure they are set appropriately.  Kermit does

its best to catch mistakes or possible mismatches, however.  For example,

you can't DIAL or CONNECT until after you have chosen a communication device

with SET PORT or SET LINE.  You also can't DIAL without first specifying a

modem type.  Furthermore, if you try to DIAL on a modem whose maximum speed

(according to Kermit's database) is less that the current speed of the

device, Kermit prints a warning message.

THE CARRIER SIGNAL

The modem is connected to the computer (perhaps conceptually) with a bunch of

wires, as explained in Appendix II, "A Condensed Condensed Guide to Serial

Communications", "Using C-Kermit".  One of these wires conveys the "carrier"

signal from the modem to the computer.  Modems are supposed to turn this

signal on only when they have a connection to another modem, and leave it off

at other times.

Some computers are set up to refuse to communicate through a serial port

unless there is a carrier signal coming in.  This is the paradox: You have to

be able to communicate with the serial port in order to give the dialing

command to the modem, but if you haven't dialed yet, there is no carrier

signal.

Kermit handles this situation as follows.  By default (unless you tell Kermit

otherwise), it commands the computer NOT to require carrier during dialing,

but to require it at (most) other times, such as during terminal emulation.

This allows Kermit (on most computers, but not all) to detect when the

connection was hung up by the other computer, and to pop back to command mode

automatically.

But if you try to communicate through a port that is not receiving the carrier

signal in any way other than by giving a DIAL command, then, depending on the

underlying computer and operating system, you are very likely to get an error

message.  This can happen, for example, if you attempt to CONNECT to the modem

and type commands at the modem's command processor.

The command that controls this sort of thing is:

SET MODEM CARRIER { AUTO, ON, OFF }

The default setting, AUTO, means not to require carrier during DIAL or

ANSWER commands, but to require it during terminal emulation.  OFF means

not to require carrier at all, so (for example) you can CONNECT to the

modem and type AT commands.  ON means to require it at all times.

CHANGING THINGS

The remaining SET MODEM commands let you adapt Kermit to unusual circumstances

or to modems that it does not know about.  Give them AFTER the SET MODEM TYPE

command, to override the values that are picked up from the database.

SET MODEM SPEED-MATCHING { ON, OFF }

Kermit assumes that if your modem is capable of speed buffering, then it it

is configured to do so, and therefore keeps its interface speed constant, no

matter what connection speed is negotiated.  On the other hand, Kermit

assumes that if your modem (according to Kermit's database) is NOT capable

of speed buffering, then its interface speed will change according to the

connection speed, and so therefore Kermit will also have to change its own

interface speed to match.  When these assumptions prove incorrect, Kermit's

speed will not match the modem's, and you will see only garbage.  Give this

command to adapt Kermit to how the modem actually behaves.

SET MODEM ERROR-CORRECTION { ON, OFF }

When this is ON and the modem has EC capability, Kermit sends the MODEM

COMMAND ERROR-CORRECTION ON string to the modem as part of the dialing

process.  When this is OFF and the modem has EC capability, Kermit sends

the MODEM COMMAND ERROR-CORRECTION OFF string.  Sometimes when an

error-correcting modem dials another modem that can't do error-correction,

the negotiations confuse the other modem so badly that the connection

can't be made.  Use SET MODEM ERROR-CORRECTION OFF to disable your modem's

error correction feature prior to dialing.

SET MODEM COMPRESSION { ON, OFF }

Works similarly to MODEM ERROR-CORRECTION { ON, OFF }.  Data compression

is almost always beneficial when it is done by the modem, at least as long

as you have an effective means of local flow control (preferably RTS/CTS)

between your computer and the modem.  If adequate flow control is lacking,

or if the compression negotiations confuse the other modem, use this

command to disable data compression.

WARNING: Certain modems come in two different models; one with

error-correction and data compression built in, and the other (normally a

cheaper model) without these features.  If you have one of the latter (which

usually include the abbreviation "RPI" or the word "controllerless" in the

technical specifications) but Kermit's database does not know this, then

dialing will fail when Kermit tries to enable these features.  To use these

modems at all, you will have to tell Kermit to:

SET MODEM ERROR-CORRECTION OFF

SET MODEM COMPRESSION OFF

and possibly also:

SET MODEM SPEED-MATCHING ON

(Also see examples below.)

SET MODEM FLOW-CONTROL { AUTO, NONE, RTS/CTS, XON/XOFF }

This tells Kermit how to configure the modem's local flow-control feature,

if any.  AUTO, the default, tries to "do the right thing" based on a

combination of the modem's capabilities (from Kermit's internal modem

database) and Kermit's FLOW-CONTROL setting.  The other options let you

override the automatic procedure, for example, when you are running Kermit

on a version of UNIX that does not let you SET FLOW RTS/CTS, but which,

nevertheless, lets you have RTS/CTS flow control by using a "special" device

name, e.g. "set line /dev/cufa", in which case Kermit did not know that

RTS/CTS could be used.  So, for example, if Kermit's FLOW-CONTROL is

NONE, but you tell Kermit to SET MODEM FLOW-CONTROL RTS/CTS, then Kermit

will send the modem its built-in MODEM COMMAND HARDWARE-FLOW string.

SET MODEM HANGUP-METHOD { MODEM-COMMAND, RS232-SIGNAL }

Tells Kermit which hangup method to use with the modem: the appropriate

modem command (e.g. ATH0) or by maninpulating RS-232 signals (e.g.  turning

off DTR).  MODEM-COMMAND is the default when the modem database includes a

command for hanging up, otherwise the RS232-SIGNAL method is used.

SET MODEM ESCAPE-CHARACTER <number>

For Hayes compatibles, Kermit normally uses plus-plus-plus as the escape

sequence.  It uses this to hang up the modem.  The escape sequence is

supposed to bring us back to the modem's command processor, and then we give

the modem-specific hangup comand, such as ATH0.  However, when the answering

modem on the other end has not been configured correctly to ignore the

escape sequence, the escape sequence is sent to the host, echoes back, and

the remote answering modem pops back to command mode, with no way to put it

back online.  If it is not in your power to reconfigure the answering modem,

then your only recourse is to use this command to change the calling modem's

escape character to something different.  <number> is the ASCII value of the

character to be used.  For Hayes compatibles, we use the guard time, three

copies of the escape character, and guard time again.  For others

(e.g. Microcom or Racal Vadic in native mode) we just send the escape

character.  This command is also handy for disabling the escape character

altogether, e.g. SET MODEM ESC 128, in case you have a TIES modem and you

need to make it transparent to all character sequences.

SET MODEM KERMIT-SPOOF { ON, OFF }

This command is used with modems that have a built-in "Kermit spoof",

in which the modem actually executes the Kermit protocol itself.  A matching

modem (e.g. Telebit) is required on the other end.  Normally Kermit can

transfer files faster without the modem's help, by using sliding windows,

long packets, etc, so the modem's Kermit spoof (if any) is disabled by

default.  Use SET MODEM KERMIT-SPOOF ON to enable the modem's Kermit spoof.

ADDING A NEW MODEM TYPE

The following commands let you define a new modem type for Kermit.

SET MODEM TYPE USER-DEFINED [ <name> ]

The optional <name> identifies one of the built-in modem types.  The

default <name> is UNKNOWN, which has no commands.  All the characteristics

of the built-in modem type are copied to the user-defined type.  For

example, if you are adding a high-speed modem that uses the Hayes command

set, choose a modem of that sort (e.g.  HAYES-HIGH-SPEED, PPI, etc.

Then you can use the following commands:

SET MODEM NAME

SET MODEM CAPABILITIES

SET MODEM MAXIMUM-SPEED

to define its basic characteristics, and then:

SET MODEM COMMAND

to change any of its commands that differ from those of the built-in modem

that you chose as a model.  These commands are now described in detail:

SET MODEM NAME <text>

Changes the descriptive name of the modem (effects SHOW MODEM only).

SET MODEM MAXIMUM-SPEED <bits-per-second>

Tell Kermit what the modem's maximum *interface* speed is, in case the

database is wrong or you are adding a new modem type.  Note: currently,

this item has no real effect, other than to cause warning messages if

Kermit's speed is higher than the modem's maximum speed when you give a

DIAL command.

SET MODEM DIAL-COMMAND <string>

This is the command used by the modem to dial a call.  The <string>

must contain "%s" (without the quotes) to show where the phone number

must be placed.  For example, for a Hayes modem, you would use

"set modem dial-command ATD%s\13".

SET MODEM CAPABILITIES [ <item> [ <item> [ ... ] ] ]

Tells Kermit the modem's capabilities in case the database is wrong or you

are adding a new modem type.  The <items> may be entered as shown in Table

II, or with their full names ("error-correction", etc), which will be

listed for you if you type "set modem capabilities ?".  AT must be set for

any modem that uses the Hayes AT command set and responses; otherwise

Kermit won't be able to read the responses; similarly for V25-COMMANDS

(ITU) for V.25bis modems.

If your user-defined modem does not use Hayes word-result, Hayes

digit-result, or V.25bis completion codes, then Kermit decides whether a

call has completed or failed based on whether the carrier signal comes on

within the DIAL TIMEOUT interval.

Now you must tell Kermit the modem-specific commands to use for each

modem function it must control.  This is done with the following command:

SET MODEM COMMAND ....

Tells Kermit the modem-specific command for various configurations and

actions, in case the database is wrong or you are adding a new modem type.

Control characters can be included in modem command strings by using

backslash notation, such as "\13" for carriage return.

Here are the SET MODEM COMMAND commands:

SET MODEM COMMAND AUTOANSWER ON <string>

The command string that puts the modem into autoanswer mode, i.e.

that makes it wait for an incoming call.

SET MODEM COMMAND AUTOANSWER OFF <string>

The command string that takes the modem out of autoanswer mode, i.e.

that puts in in originate mode for making calls.

SET MODEM COMMAND COMPRESSION ON <string>

The command string that enables the modem to negotiate data compression

with the other modem.

SET MODEM COMMAND COMPRESSION OFF <string>

The command string that turns off the modem's data compression feature

and disables data-compression negotiation with the other modem.

SET MODEM COMMAND DIAL-MODE-STRING <string>

SET MODEM COMMAND DIAL-MODE-PROMPT <string>

Command (if any, normally none) used to put modem into dialing mode,

and the prompt (if any, normally none) issued by the modem when it is

in dialing mode.

SET MODEM COMMAND ERROR-CORRECTION ON <string>

The command string that enables the modem to negotiate error correction

with the other modem.

SET MODEM COMMAND ERROR-CORRECTION OFF <string>

The command string that turns off the modem's error correction feature

and disables error-correction negotiation with the other modem.

SET MODEM COMMAND HANGUP-COMMAND <string>

The command string that tells the modem to hang up the telephone connection.

SET MODEM COMMAND HARDWARE-FLOW <string>

The command string that enables local hardware (RTS/CTS) flow control

in the modem.

SET MODEM COMMAND SOFTWARE-FLOW <string>

The command string that enables local software (Xon/Xoff) flow control

in the modem.  This should be used only if RTS/CTS is not available.

SET MODEM COMMAND NO-FLOW-CONTROL <string>

The command string to disable local flow control in the modem.

SET MODEM COMMAND INIT-STRING <string>

The command string that should be used to initialize the modem.

SET MODEM COMMAND PULSE <string>

The command string that sets the modem's dialing method to pulse.

SET MODEM COMMAND TONE <string>

The command string that sets the modem's dialing method to tone.

For Hayes compatibles, the MODEM INIT-STRING should contain Q0 to ensure the

modem gives result codes for commands.  They can be either numeric or word

result codes; Kermit handles both automatically.  You can also make the modem

echo (E1) or not echo (E0) commands -- Kermit should work either way.  The

init string should contain all commands that are not affected by other SET

MODEM settings, particularly the command to enable BREAK transparency, the

command to enable modulation negotiation, a command to enable a fairly high

level of dial result codes (e.g. X4).

Example 1: Best Data 14400 bps Fax Modem V.32bis/V.17

SET MODEM USER-DEFINED PPI                ; Use PPI as the template

SHOW MODEM                                ; To see what you've got

Now make any needed changes:

SET MODEM NAME Best Data 1442FTX          ; or whatever

;; SET MODEM CAPABILITIES ...             ; (not needed, same as PPI)

SET MODEM MAXIMUM-SPEED 57600             ; or whatever, if necessary

SET MODEM COMMAND INIT ATF1N1W1Y0&C1&D2X4S37=0\13  ; init-string

SET MODEM COMMAND HARD AT&K3\\G1\13       ; hardware flow on

SET MODEM COMMAND SOFT AT&K4\\G1\13       ; software flow on

SET MODEM COMMAND NO-F AT&K0\\G0\13       ; no flow control

SET MODEM COMMAND ERROR ON  AT&Q6\\N3\13  ; error-correction enabled

SET MODEM COMMAND ERROR OFF AT&Q0\\N1\13  ; error-correction disabled

SET MODEM COMMAND COMPRESS ON  AT%C3\13   ; compression enabled

SET MODEM COMMAND COMPRESS OFF AT%C0\13   ; compression disabled

Example 2: SupraExpress 144i Fax/Modem

This one is an RPI model, and so lacks error correction and data

compression, but the commands and other features are the same as the

built-in Supra type:

SET MODEM-TYPE USER-DEFINED SUPRA

SET MODEM NAME SupraExpress 144i Fax/Modem RPI

SET MODEM CAPABILITIES AT SB HWFC SWFC

or if speed-buffering doesn't work (since, in general, it depends on an

error-correcting protocol between the modems):

SET MODEM CAPABILITIES AT HWFC SWFC

By removing the EC and DC capabilities, we prevent Kermit from sending

any EC- or DC-related commands to the modem, which, in this model, would

cause an error.

--------------------------------

APPENDIX I - MODEM COMMAND LIST

SET MODEM TYPE <name>

SET MODEM CARRIER { AUTO, ON, OFF }

SET MODEM CAPABILITIES { AT, DC, EC, HWFC, ITU, KS, SB, SWFC, TB }

SET MODEM MAXIMUM-SPEED { 1200, 2400, ..., 57600, 115200, ... }

SET MODEM ERROR-CORRECTION { ON, OFF }

SET MODEM COMPRESSION { ON, OFF }

SET MODEM SPEED-MATCHING { ON, OFF }

SET MODEM FLOW-CONTROL { AUTO, NONE, RTS/CTS, XON/XOFF }

SET MODEM KERMIT-SPOOF { ON, OFF }

SET MODEM ESCAPE-CHARACTER <number>

SET MODEM COMMAND AUTOANSWER ON <string>

SET MODEM COMMAND AUTOANSWER OFF <string>

SET MODEM COMMAND COMPRESSION ON <string>

SET MODEM COMMAND COMPRESSION OFF <string>

SET MODEM COMMAND ERROR-CORRECTION ON <string>

SET MODEM COMMAND ERROR-CORRECTION OFF <string>

SET MODEM COMMAND HANGUP-COMMAND <string>

SET MODEM COMMAND HARDWARE-FLOW <string>

SET MODEM COMMAND SOFTWARE-FLOW <string>

SET MODEM COMMAND NO-FLOW-CONTROL <string>

SET MODEM COMMAND INIT-STRING <string>

SET MODEM COMMAND PULSE <string>

SET MODEM COMMAND TONE <string>

--------------------------------------------------------------------

APPENDIX II - DIFFERENCES BETWEEN NEW AND OLD MODEM-RELATED COMMANDS

This applies to the changes that occurred in C-Kermit 5A(192) and other

programs based upon it.

1. SET MODEM <name> became SET MODEM TYPE <name>.  SET MODEM <name> is still

accepted, but ?-help doesn't work; completion, and editing don't work

very well.

2. Many additional modem types were added, mostly newer high-speed models.

3. SET MODEM TYPE <name> fills in numerous dial-, modem-, and communications

parameters from the modem database automatically.  Ditto for the -m <name>

command-line option.

4. The former SET DIAL command was divided into SET MODEM and SET DIAL.

Items that related to modems, but not to dialing, became SET MODEM

commands.  Items that related to dialing, but not modems, remained SET

DIAL commands.  Most of the old SET DIAL forms, however, are still

accepted.

Here is the list of old and new modem-related commands.  All of the old

commands are still accepted, and work as documented in "Using C-Kermit", but

they are invisible -- i.e. they are not revealed by "?" or "help set modem"

or "help set dial".

New commands:                          Equivalent old commands, if any:

SET MODEM TYPE                         SET MODEM

SET MODEM CARRIER                      SET CARRIER

SET MODEM CAPABILITIES

SET MODEM DIAL-COMMAND                 SET DIAL DIAL-COMMAND

SET MODEM MAXIMUM-SPEED

SET MODEM ERROR-CORRECTION             SET DIAL MNP

SET MODEM COMPRESSION                  SET DIAL MNP

SET MODEM SPEED-MATCHING               SET DIAL SPEED-MATCHING

SET MODEM FLOW-CONTROL

SET MODEM KERMIT-SPOOF                 SET DIAL KERMIT-SPOOF

SET MODEM ESCAPE-CHARACTER

SET MODEM COMMAND AUTOANSWER

SET MODEM COMMAND COMPRESSION

SET MODEM COMMAND ERROR-CORRECTION

SET MODEM COMMAND HANGUP-COMMAND

SET MODEM COMMAND HARDWARE-FLOW

SET MODEM COMMAND SOFTWARE-FLOW

SET MODEM COMMAND NO-FLOW-CONTROL

SET MODEM COMMAND INIT-STRING

SET MODEM COMMAND PULSE

SET MODEM COMMAND TONE

There are also many new SET DIAL commands, which are described in the

section on dialing.

(End)