Larry Gritz once wrote:

HOW TO FIND GAMMA ON YOUR MONITOR

First, make the room dark and crank up the monitor contrast and

brightness.  Lower the brightness gradually until the "background" is

no longer noticable (when it just fades from view, stop turning the

knob).  Then set the contrast to the highest it can get without

causing any distortion or other artifacts.

Now look at the file gamma.png, which is from Greg Ward [WvS: the TIFF

version was from Ward, I converted it to PNG]. This displays a scale

that lets you know the gamma of your monitor.  Whichever labeled

"swatch" on the right matches the left side, that is your gamma.

---

Willem van Schaik

gwillem@ntuvax.ntu.ac.sg